# Convert the Sensor Reading into JSON

Before connecting to FastAPI or the cloud, you should package the working sensor data into a structured JSON message.

# Example

If the sensor currently displays:

Heart rate: 76 BPM
SpO2: 98%

Convert it to:

{
  "device_id": "esp32-01",
  "heart_rate_bpm": 76,
  "spo2_percent": 98
}

# ESP32 code

Install the ArduinoJson library, then add:

#include <ArduinoJson.h>

Inside loop(), after obtaining the sensor readings:

StaticJsonDocument<200> data;

data["device_id"] = "esp32-01";
data["heart_rate_bpm"] = heartRate;
data["spo2_percent"] = spo2;

String jsonMessage;
serializeJson(data, jsonMessage);

Serial.println(jsonMessage);

You should replace heartRate and spo2 with the variable names used in the sensor's working code.

# Expected Serial Monitor output

{"device_id":"esp32-01","heart_rate_bpm":76,"spo2_percent":98}

# Completion check

The immediate step is complete when the Serial Monitor displays valid JSON containing live sensor values. Now take a screenshot because it provides evidence that their hardware data is ready to be sent to the REST API.