# Create a Local FastAPI Endpoint

The next immediate step is to create a local FastAPI endpoint that receives the JSON sensor data. You should first test it using the same example JSON before connecting the ESP32.

# 1 Install FastAPI and Uvicorn

In the computer’s terminal, run:

pip install fastapi uvicorn

# 2 Create a file named main.py

from fastapi import FastAPI
from pydantic import BaseModel

app = FastAPI()

class SensorReading(BaseModel):
    device_id: str
    heart_rate_bpm: float
    spo2_percent: float

@app.post("/sensor-data")
def receive_sensor_data(reading: SensorReading):
    print(reading.model_dump())
    return {
        "status": "received",
        "data": reading.model_dump()
    }

# 3 Run the service

From the folder containing main.py, run:

uvicorn main:app --reload

# 4 Test the endpoint in the browser

Open http://127.0.0.1:8000/docs in a browser. Expand POST /sensor-data, click Try it out, enter the following JSON, and click Execute:

{
  "device_id": "esp32-01",
  "heart_rate_bpm": 76,
  "spo2_percent": 98
}

# Completion check

You should receive an HTTP 200 response containing "status": "received" with the submitted values. You should take a screenshot of this response as evidence that their REST API can accept the sensor message.