# Send Live ESP32 Data to FastAPI

The immediate next step is to connect the ESP32 to Wi-Fi and send its live JSON sensor readings to the local FastAPI endpoint.

This connects the two parts you have already tested: the sensor’s JSON output and the API that receives it.

# 1 Connect both devices to Wi-Fi

Connect the ESP32 and computer to the same Wi-Fi network. The network must allow devices to communicate.

# 2 Run FastAPI to accept ESP32 connections

uvicorn main:app --host 0.0.0.0 --port 8000 --reload

# 3 Find the computer’s local IP address

On Windows, run ipconfig and find the Wi-Fi adapter’s IPv4 address.

# 4 Send the JSON using an HTTP POST request

Program the ESP32 to send an HTTP POST request containing jsonMessage to:

http://<computer-IP>:8000/sensor-data

For example:

http://192.168.1.10:8000/sensor-data

Set the request’s Content-Type to application/json. Use the computer’s local IP address; 127.0.0.1 on the ESP32 refers to the ESP32 itself.

# Completion check

The ESP32’s Serial Monitor shows HTTP 200, and the computer’s FastAPI terminal displays the live sensor values received from the ESP32. Yous should capture both as evidence.